[image: image14.png]




Strada 

Evolution / Rapide
ELOCK 

Replacement Procedure

[image: image1.png]


                                
Safety:
Parkeon designs its product to limit any risk to users or operators regarding electrical power sources, high temperatures, fire mechanics, radiation or chemicals. Parkeon products have CE mark. They comply with the following European directives: 93/68/EEC, 89/336/EEC (EMC) for all versions and 73/23 EEC (LV) for mains or public lighting power supply versions.

Follow basic precautions below for safety of personnel and environment. This necessarily incomplete list draws attention to different potential risk due to incorrect operations or lack of attention.

[image: image2.png]


 Electricity:

· All operations on product require appropriate applicable training. Only properly trained personnel may complete operations on mains power supply. Disconnect mains throughout all these operations.
· Protect against electrical shocks with proper power and earth connections. Check installation of electrical parts complies with national standards. Check protective conductor (earth cable) is in good condition. Obtain a certificate signed by contracted enterprise.

· Respect all earth cabling between product elements to comply with EMC emission and immunity standards.

· Never undertake operations on a mains power supply or telephone connected product during stormy weather. Protect exposed electrical cables, electronic or electromagnetic sub-assemblies when it is raining.

· Always switch off electricity before doing certain product operations as described in this guide. Isolate mains power supply with cut-out switch and switch off low voltage with main board on/off switch.

· Always wear anti-static bracelet when undertaking operations on electronic sub-assemblies.

[image: image3.png]


 Mechanical Parts:
· Always remove metal jewelry (rings, bracelets or wrist watches) before operating on or near to electrical (power supply, battery, cabling) or mobile parts of product.

· Keep hands away from mobile, moving or cutting parts, from door hinges or locks, and from gear wheels or blades.

· Pay attention to injury from open door or other extruding or overhanging parts of product.

· Only properly trained personnel may move pedestal, coin box or housing. Two people are necessary. They must wear safety shoes.
· Use correct good condition tools in their intended manner.

[image: image4.png]


 Temperature:
· Always wait several minutes after switching off power before working near to hot areas of intermittent heating devices.

[image: image5.png]


 Batteries:
· Always respect battery and dry cell polarity.

· Never discard burn or open used buffer or dry cell batteries. Use appropriate recycling circuit as recommended by local health and safety authorities.

· Even though batteries are fully charged when new, recharge stored batteries every six months.

· Batteries of terminals in operation should be changed after every two years.
Read instructions fully before installing the Elock.
Inside view without collection door
[image: image6.png]



Prior to replacing the Elock you must perform the following steps if possible.
1. Perform a complete coin collection and keep collection ticket with canister.

2. Perform an OSP 503 “bank online maintenance” (if applicable)

3. Perform an OSP 57 (cap file transfer)

4. Perform a second coin collection but DO NOT close the collection door. 

5. Power off the main board.

6. Disconnect the battery cable.

7. Disconnect the solar or mains charger board cable (which ever one is used).

Collection Door Removal:
8. Open the Maintenance door and lift up on the locking bar, which is located on the bottom of the left wall of maintenance compartment in the front left corner. This locking bar secures the securing screw for the collection doors left side screw.
[image: image14.png]
[image: image7.png]



9. Unscrew the 2 screws going through the rotation bearings. If the left screw tightens as you unscrew it, you must lift up higher on the locking bar.
10. Using the flat blade of a screwdriver against the wall of the collection door pry out on the rotation bearings to remove them

[image: image8.png]



11. Open the collection door until it is lying in the horizontal position.

12. Lift up and pull the collection door out of the machine. Be mindful not to damage the paint.

[image: image9.png]



Elock Removal:
13. With the main board power off and an anti static strap on disconnect the electronic lock connectors.
[image: image10.png]SRS

=





14. Hold the Elock and remove the securing screw with a 5mm Allen key.

[image: image11.png]



15. Take the Elock out of its housing.
[image: image12.png]



16. Install and fix the new Elock and connect all of the connectors

[image: image13.png]


IMPORTANT:

· Never switch on the meter with out the collection door in place.

· Do not close collection door without coin box.

· Before placing the coin box in the meter confirm that the once only system at the top is set.
Collection Door Installation:
1. Holding the collection door in the horizontal position lift up slightly to clear the door pins.

2. Rotate the collection door to approximately 3-4” open and reinstall the rotation bearings on the hinge pins.

3. Fasten the door using the 2 Screws removed previously (remember to lift up on the locking bar) to tighten the left screw.

4. Test the operation of the door.

5. Install the collection canister and retest the door operation (resetting the canister afterwards)

6. Verify the Collection door is closed and the power switch is in the off position.

7. Reconnect the Solar / Mains power supply cable and the battery cable.

8. Power up the meter and perform an OSP 80.

9. Verify Collection door is secured.

Once the Elock is initialized you will need to program the combination.
1. Enter the collection menu by pressing and holding the Red and Green buttons on the front of the maintenance door. 

Hold both buttons down until you see the collection/maintenance menu.

2. Place the Blue programming key on the electronic key interface:

Message “Programming”
3. When the message “Standing by for collection key” is displayed place the Black collection key on the electronic key interface:

4. When the Green Indicator light comes on above the collection door perform a collection afterwards the meter will display “programming complete”.
	Date of last modification: 4/10/2026
	Creation Date: 4/10/2026

	This document belongs to Parkeon. It cannot be copied, executed or distributed to any body without written authorization from us

	Page: 

	Strada Evolution / Rapide Elock Replacement Procedure



_1290318865

_1286858942.bin

